SKILLS SCIENCE ACADEMY

Computer Science — First Year

CHAPTER 1

Computer Systems

Comprehensive Student Notes

Topics Covered:

1.1  Data Representation in a Digital Computer

1.2  Analog and Digital Signals

1.3  Digital Logic and Logic Gates

1.4  Software Development Life Cycle (SDLC)

1.5  Network Topology

1.6  Cybersecurity

# 1.1  Data Representation in a Digital Computer

## Overview

A digital computer can be thought of as a huge collection of tiny electronic switches. Each switch can only be in one of two positions:

- ON — electricity flows — represents 1

- OFF — no electricity flows — represents 0

Computers only understand these two states, 0 and 1. Everything a computer processes — photos, videos, text, and games — is ultimately represented as a pattern of 0s and 1s.

### What is a Bit?

A bit is a single binary digit — either a 0 or a 1.

Analogy:  If a light switch is a bit, then a byte (8 bits) is like a row of 8 light switches. With 8 switches, 256 different ON/OFF combinations are possible (2⁸ = 256).

### What is Binary Code?

When bits are grouped together, they form binary code. For example:

- 1000100 may represent the letter D

- 1000001 may represent the letter A

### What is ASCII?

ASCII (American Standard Code for Information Interchange) is a standard that assigns a unique number to every character.

| Character | ASCII Code |
| --- | --- |
| A | 65 |
| B | 66 |
| a | 97 |
| 0 | 48 |
| Space | 32 |

Key facts about ASCII:

- Uses 7 or 8 bits per character

- Can represent 128 characters (2⁷ = 128)

- Includes uppercase letters, lowercase letters, digits 0–9, and special symbols (%, $, &, #, +, etc.)

Exam Tip:  “How many bits does ASCII use?” → 7 or 8 bits. “How many characters?” → 128.

### What is an Alphanumeric Code?

Any code that represents letters and numbers (and special characters) is called an alphanumeric code. ASCII is the most common example.

# 1.2  Analog and Digital Signals

## Overview

An analog signal is a smooth, continuous wave that can take any value, such as a sine wave. A digital signal is a square wave that jumps between just two levels, HIGH and LOW — like a light switch that is either fully ON or fully OFF.

### Key Differences

| Feature | Analog Signals | Digital Signals |
| --- | --- | --- |
| What it looks like | Smooth, continuous wave | Square wave (ON/OFF) |
| Values possible | Infinite values in a range | Only 0 or 1 |
| Noise resistance | Easily affected by noise | Very resistant to noise |
| Long distance | Degrades over distance | Travels far with less degradation |
| Copying | Quality loss when copied | Perfect copies every time |
| Examples | Cassette tapes, VHS, human speech | MP3, JPEG, MP4, computer data |

### Real-Life Examples

Analog:

- Old cassette tape music

- VHS video tapes

- Your voice when you speak

- Mercury thermometer

Digital:

- MP3 songs on your phone

- Photos taken with a digital camera

- YouTube videos

- Data on your laptop

Why digital is better:  Copying an analog cassette tape repeatedly reduces its quality each time. Copying a digital MP3 file, even 1000 times, produces an exact copy every time — this is why digital formats are preferred.

# 1.3  Digital Logic and Logic Gates

## Overview

Logic gates are the tiny decision-makers inside a computer. They take one or more inputs, each either 0 (OFF) or 1 (ON), and produce an output based on a fixed rule. For example, an AND gate turns ON only if all its inputs are ON, an OR gate turns ON if any input is ON, and a NOT gate always produces the opposite of its input.

### 3. NOT Gate (Inverter)

Rule: Output is the opposite of the input.

Expression: F = x̄ (x-bar)

| x | F |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

Analogy:  A NOT gate behaves like a rebellious teenager — whatever you tell it, it does the opposite.

### 1. AND Gate

Rule: Output is 1 only when ALL inputs are 1.

Expression: F = x · y (or simply xy)

| x | y | F |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Analogy:  Two switches wired in series — both must be ON for the light to turn on.

### 2. OR Gate

Rule: Output is 1 when ANY input is 1.

Expression: F = x + y

| x | y | F |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Analogy:  Two switches wired in parallel — either one can turn on the light.

### 4. NAND Gate (NOT + AND)

Rule: Output is 0 only when ALL inputs are 1 (opposite of AND).

Expression: F = (xy)′

| x | y | F |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

### 5. NOR Gate (NOT + OR)

Rule: Output is 0 when ANY input is 1 (opposite of OR).

Expression: F = (x + y)′

| x | y | F |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

### 6. XOR Gate (Exclusive OR)

Rule: Output is 1 when the inputs are DIFFERENT.

Expression: F = x ⊕ y

| x | y | F |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Analogy:  “We agree to disagree” — output is 1 only when the inputs disagree.

### 7. XNOR Gate (Exclusive NOR)

Rule: Output is 1 when the inputs are the SAME (opposite of XOR).

Expression: F = (x ⊕ y)′

| x | y | F |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

### What is a Truth Table?

A truth table shows the output of a logic gate or circuit for every possible combination of inputs.

Rules for constructing a truth table:

- List all input combinations (count in binary: 00, 01, 10, 11 for two inputs)

- Add columns for any intermediate working

- Add a final output column

- Fill in the results based on the gate's rule

Exam Tip:  For 2 inputs → 4 rows. For 3 inputs → 8 rows. For n inputs → 2ⁿ rows.

### Boolean Identities (Laws)

These identities are always true and are used to simplify complex Boolean expressions.

| Identity | Name |
| --- | --- |
| A + 0 = A | Identity Law |
| A · 1 = A | Identity Law |
| A + 1 = 1 | Domination Law |
| A · 0 = 0 | Domination Law |
| A + A = A | Idempotent Law |
| A · A = A | Idempotent Law |
| A + A′ = 1 | Complement Law |
| A · A′ = 0 | Complement Law |
| (A′)′ = A | Double Negation |

Model Paper Q1(49):  Simplify A + A·B → A(1 + B) = A · 1 = A

### Principle of Duality

The dual of any Boolean expression is obtained by:

- Swapping AND (·) and OR (+)

- Swapping 0 and 1

| Original | Dual |
| --- | --- |
| 1 = 0 | 0 = 1 |
| A · 0 = 0 | A + 1 = 1 |
| A + B = B + A | A · B = B · A |
| A + (B · C) = (A + B) · (A + C) | A · (B + C) = (A · B) + (A · C) |

Exam Tip:  Duality is simply “swap AND/OR and swap 0/1.” The resulting expression remains valid.

### Karnaugh Map (K-Map)

A Karnaugh Map is a visual tool used to simplify Boolean expressions without lengthy algebra. Using a K-Map reduces the number of gates required, which lowers the cost of a circuit.

Rules for K-Map simplification:

- Plot 1s — place a 1 in the cell corresponding to each term in the function

- Group 1s into groups of 2, 4, 8, etc. (powers of 2)

- Groups must be adjacent — horizontal or vertical (never diagonal)

- Groups can overlap and can wrap around the edges of the map

- Eliminate any variable that appears in both normal and complemented form within a group

- Write the simplified expression as the sum of the remaining variables

### Worked Example: Simplify F = A′B + AB′ + AB

Step 1 — Plot the 1s on a 2-variable K-Map:

| A \ B | B′ | B |
| --- | --- | --- |
| A′ | 0 | 1 (A′B) |
| A | 1 (AB′) | 1 (AB) |

Step 2 — Group the adjacent 1s:

- Group 1: A′B and AB (vertical pair) → eliminates A → gives B

- Group 2: AB′ and AB (horizontal pair) → eliminates B → gives A

Step 3 — Final simplified answer:

F = A + B

Exam Tip:  K-Maps are a guaranteed extended-response question (ERQ) in the exam. Practice 3-variable K-Maps thoroughly.

### Uses of Logic Gates

Logic gates are used in:

- Memory circuits (flip-flops, latches)

- Calculators (arithmetic operations)

- Traffic light controllers

- Robots (movement and decision-making)

- Security systems (alarms, access control)

- Home automation (smart lighting, temperature control)

- Medical monitoring devices

- Aerospace navigation and autopilot systems

# 1.4  Software Development Life Cycle (SDLC)

## Overview

The Software Development Life Cycle (SDLC) is the step-by-step process followed to build software. Just as building a house requires planning, designing, constructing, and inspecting, building software follows a similarly structured approach.

### The Eleven Phases of SDLC

1. Defining the Problem  →  2. Planning  →  3. Feasibility Study  →  4. Analysis  →  5. Requirement Engineering  →  6. Design  →  7. Development/Coding  →  8. Testing/Verification  →  9. Deployment  →  10. Documentation  →  11. Maintenance

### Phase 1: Defining the Problem

Clearly identify what problem the software will solve. Example: “We need a Students' Examination System that handles exam-taking and result generation.”

### Phase 2: Planning

Set goals and estimate resources such as people, money, and time, then create a project plan. Example: “We need 3 developers, 2 testers, 6 months, and Rs. 5,00,000.”

### Phase 3: Feasibility Study

A feasibility study checks whether the project is actually achievable.

| Type | Question |
| --- | --- |
| Technical | Do we have the required technology? |
| Economic | Can we afford it? Will the benefits outweigh the costs? |
| Operational | Will it work well within our organization? |
| Legal | Is it legal? Does it comply with regulations? |
| Schedule | Can it be completed on time? |

Exam Tip:  Legal feasibility refers to compliance with laws and regulations (MCQ Q1(52)).

### Phase 4: Analysis

Study the existing system and consult users to understand their needs. Key questions: Can this be built with available resources? Will it improve the organization? Does the existing system need replacing?

### Phase 5: Requirement Engineering

This phase involves gathering, validating, and managing requirements. Common techniques for gathering requirements include:

- Interviews — direct conversation with stakeholders

- Surveys/Questionnaires — written questions for large groups

- Observation — watching users in their working environment

- Document Analysis — reviewing existing documents

### Phase 6: Design

This phase creates the blueprint of the system, including algorithms, flowcharts, database structure, and user interface.

Flowchart symbols:

| Symbol | Name | Function |
| --- | --- | --- |
|  | Terminal | Start / End |
|  | Input/Output | Data entry or display |
|  | Decision | Yes/No question |
|  | Process | Calculations |
|  | Flow Line | Direction |
|  | Connector | Links parts of the flowchart |

### Phase 7: Development/Coding

The actual code is written using programming languages such as Python, C++, or Java.

### Phase 8: Testing/Verification

This phase finds and fixes bugs (errors) in the software.

| Black Box Testing | White Box Testing |
| --- | --- |
| Tester does not see the internal code | Tester knows the internal code |
| Tests input → output behavior | Tests internal logic and paths |
| Focus on functionality | Focus on code quality |

Exam Tip:  White-box testing means examining the internal code structure (MCQ Q1(35)).

### Phase 9: Deployment/Implementation

This phase involves installing the software, training users, and switching from the old system to the new one.

| Method | Description | When to Use |
| --- | --- | --- |
| Direct | Old system replaced immediately | Small systems, low risk |
| Parallel | Both systems run together for a while | High-risk, critical systems |
| Phased | Gradual transition, part by part | Large systems |
| Pilot | Tested with a small group first | New or innovative systems |

Exam Tip:  The parallel method means the old and new systems run simultaneously (MCQ Q1(23)).

### Phase 10: Documentation

User manuals, technical guides, and system documentation are created in this phase. Good documentation supports maintenance, training, and future development.

### Phase 11: Maintenance/Support

After release, this phase covers fixing bugs, updating features, and adapting to changing needs.

- Corrective — fixes errors found after release

- Adaptive — updates the system for new environments

- Perfective — improves performance or adds features

### Software Development Models

Waterfall Model:

A sequential, linear approach in which each phase must finish before the next begins (Requirements → Design → Development → Testing → Deployment → Maintenance).

| Advantages | Disadvantages |
| --- | --- |
| Easy to understand and implement | Inflexible to changes |
| Clear milestones | Long delivery time |
| Well documented | Late testing means late bug detection |
| Good for projects with fixed requirements | High risk if requirements change |

Best for: building construction and weather-forecasting systems, where requirements are fixed.

Agile Model:

An iterative, incremental approach in which work is completed in short cycles called “sprints.”

| Advantages | Disadvantages |
| --- | --- |
| Flexible and adaptable | Depends on customer availability |
| Continuous client involvement | Scope creep is possible |
| Early delivery of working software | Less documentation |
| Regular feedback and improvement | Not ideal for fixed requirements |

Best for: web applications, mobile apps, and learning management systems, where requirements change frequently.

Exam Tip:  Waterfall suits fixed requirements; Agile suits changing requirements (ERQ Q1).

# 1.5  Network Topology

## Overview

Network topology refers to the arrangement or layout of computers and devices in a network — much like arranging desks in a classroom, where different layouts bring different advantages and drawbacks.

### Bus Topology

Layout: All devices are connected to a single main cable (the “bus”).

| Advantages | Disadvantages |
| --- | --- |
| Cheap — only one cable is needed | If the main cable breaks, the whole network fails |
| Easy to install | Congestion increases with more devices |
| Fast for small networks | Difficult to troubleshoot |

Used in: Small office and home networks

### Star Topology

Layout: All devices connect to a central hub or switch.

| Advantages | Disadvantages |
| --- | --- |
| Reliable — one device failure does not affect others | If the central hub fails, the whole network fails |
| Easy to install and manage | More expensive — requires a hub/switch |
| High performance |  |

Used in: Large offices, campus networks, homes

### Ring Topology

Layout: Devices are connected in a circular loop; data travels in one direction.

| Advantages | Disadvantages |
| --- | --- |
| High reliability with a dual-ring option | If one device or connection breaks, the whole ring fails |
| Cost-effective | Limited bandwidth |
| No data collisions | Difficult to troubleshoot |

Used in: LANs and fibre-optic networks (FDDI)

### Mesh Topology

Layout: Every device connects directly to every other device.

| Advantages | Disadvantages |
| --- | --- |
| Highly reliable — multiple paths available | Very expensive — requires many cables |
| Redundant — data can take another route if one path fails | Complex to set up |
| Scalable | Difficult to maintain |

Used in: Military, corporate data centres, critical systems

### Tree Topology

Layout: A hierarchical layout, like a tree with branches — multiple star networks connected together.

| Advantages | Disadvantages |
| --- | --- |
| Scalable | Complex installation |
| Reliable | Difficult to troubleshoot |
| Cost-effective |  |

Used in: Large corporate environments

### Hybrid Topology

Layout: A combination of two or more topologies.

| Advantages | Disadvantages |
| --- | --- |
| Flexible | Complex |
| Reliable | Expensive |
| Scalable | Difficult to troubleshoot |
| Secure |  |

Used in: Large enterprise networks

### Scalability and Reliability Comparison

| Topology | Scalability | Reliability |
| --- | --- | --- |
| Bus | Low | Low |
| Star | Moderate | Good |
| Ring | Moderate | High |
| Mesh | High | Very High |
| Hybrid | High | Varies |

Exam Tip:  Mesh topology is the most reliable due to multiple paths. Star is the most common. Bus is the cheapest.

### Cloud Computing

Cloud computing means using computing resources — servers, storage, and software — over the internet instead of owning them directly.

Service Models:

| Model | What It Provides | Example |
| --- | --- | --- |
| IaaS | Virtual machines, storage, networks | AWS EC2 |
| PaaS | Development platforms and tools | Google App Engine |
| SaaS | Software applications | Gmail, Microsoft 365 |

Deployment Models:

| Model | Description |
| --- | --- |
| Public | Open to everyone (e.g., AWS, Azure) |
| Private | Used by a single organization only |
| Hybrid | A mix of public and private cloud |
| Community | Shared by organizations with common needs |

Scalability:

- Horizontal (Scaling Out): adding more servers

- Vertical (Scaling Up): upgrading an existing server (more RAM, CPU)

Reliability: high availability, redundancy, fault tolerance, and disaster recovery.

Exam Tip:  SaaS delivers software over the internet (e.g., Gmail). Private cloud is the most secure. Horizontal scaling means adding more machines.

# 1.6  Cybersecurity

## Overview

Cybersecurity is the practice of protecting computers, networks, and data from digital attacks — much like locking doors and installing alarms to protect a home.

### Why is Cybersecurity Important?

- Protects sensitive data such as personal and financial information

- Prevents cyber attacks such as malware, phishing, and DDoS

- Safeguards critical infrastructure such as power grids and hospitals

- Maintains business continuity by avoiding downtime

- Ensures legal compliance with regulations

- Protects national security, including government systems

- Preserves the privacy of personal information

### Types of Cyber Threats

| Threat | Description | Example |
| --- | --- | --- |
| Virus | Attaches to files and spreads when opened | Email attachments |
| Worm | Self-replicating; spreads automatically | Network infection |
| Trojan | Disguised as legitimate software | Fake antivirus |
| Ransomware | Encrypts files and demands payment | WannaCry |
| Spyware | Collects user information secretly | Keyloggers |
| Phishing | Fake emails/websites used to steal information | Fake bank email |
| DoS/DDoS | Overwhelms a system with traffic | Website crash |
| Insider Threat | An employee misuses their access | Data theft |

Exam Tip:  A worm spreads automatically without any user action (MCQ Q1(5)).

### Protection Methods

| Method | How It Works |
| --- | --- |
| Strong Passwords | 12+ characters, mixing upper/lowercase, numbers, and symbols |
| Software Updates | Patch known security holes |
| Two-Factor Authentication (2FA) | Requires two verification methods (password + OTP) |
| Suspicious Email Awareness | Avoid clicking unknown links |
| Education | Stay informed about current threats |
| Firewalls | Filter incoming and outgoing traffic |
| Antivirus | Detect and remove malware |
| Encryption | Convert data into an unreadable format |
| Backup | Keep regular copies of data |
| Cryptography | Secure communication |

Exam Tip:  2FA combines something you know (password) with something you have (OTP).

### Encryption

Encryption is the process of converting readable data (plaintext) into unreadable data (ciphertext) to protect it.

Process: Plaintext → [Encryption Algorithm + Key] → Ciphertext → [Decryption Algorithm + Key] → Plaintext

Symmetric Encryption:

- Uses one key for both encryption and decryption

- Fast and efficient

- Challenge: securely sharing the key

Analogy:  A single key that both locks and unlocks the same box.

Asymmetric Encryption:

- Uses two keys: a public key (encrypts) and a private key (decrypts)

- Slower, but more secure

- Solves the key-distribution problem

Analogy:  Anyone can place a letter in your mailbox using the public key, but only you hold the private key needed to open it.

| Symmetric | Asymmetric |
| --- | --- |
| Single shared key | Public and private key pair |
| Fast | Slow |
| Same key for both operations | Different keys for encryption and decryption |
| Key distribution is difficult | Key distribution is easier |
| Good for large data volumes | Good for small data / key exchange |

Exam Tip:  Symmetric encryption uses one key and is fast. Asymmetric encryption uses two keys and is more secure (ERQ Q2).

# Summary — Key Takeaways for the Exam

### Must-Know Definitions

- Bit — a single binary digit

- ASCII — a 7/8-bit code representing 128 characters

- Logic Gate — the basic building block of digital circuits

- Truth Table — shows all input/output combinations

- Duality — swap AND/OR and swap 0/1

- K-Map — a visual Boolean-simplification tool

- SDLC — the 11-phase software development process

- Waterfall — a sequential development model

- Agile — an iterative development model

- Topology — the arrangement of a network

- Cloud Computing — internet-based computing resources

- Cybersecurity — protection from digital attacks

- Symmetric Encryption — uses one key

- Asymmetric Encryption — uses two keys (public/private)

- 2FA — two-factor authentication

### Must-Know Comparisons

- Analog vs Digital signals

- Waterfall vs Agile

- Symmetric vs Asymmetric encryption

- Bus vs Star vs Ring vs Mesh topology

- Black box vs White box testing

- Horizontal vs Vertical scaling

### Must-Know Lists

- The 7 logic gates and their truth tables

- The 11 SDLC phases

- 4 deployment methods

- 4 cloud deployment models

- 3 cloud service models

- Types of cyber threats

- Methods of protection

### Must-Know Skills

- Draw truth tables for any logic gate

- Simplify Boolean expressions using identities

- Simplify expressions using K-Maps (2 and 3 variables)

- Draw logic circuits from a Boolean expression

- Identify SDLC phases in a given scenario

- Choose the appropriate network topology for a situation

- Compare encryption methods

These notes cover Unit 1 comprehensively. Practice K-Maps and SDLC phases especially — they are guaranteed exam topics.